, SAFRAN

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

Cheltenham Road, Gloucester, GL2 9QH, England

Telephone: +44 (0) 1452 712424

www.safran-landing-systems.com

MAIN LANDING GEAR LEG

PART NUMBER

201587001, 201587002

201587003, 201587004

201587005, 201587006

201587007, 201587008

COMPONENT MAINTENANCE MANUAL
WITH

ILLUSTRATED PARTS LIST

STATEMENT OF INITIAL CERTIFICATION

This manual complies with British Civil Airworthiness Requirements, Section A, Chapter A5-3.

CAA Approval No. DAI/1018/39

NOTE: The above certification does not apply to revisions or amendments made after the date of initial certification by other Approved
Organisations. Revisions or Amendments made by other Approved Organisations must each be separately certified and recorded
on separate record sheets.

The technical data in this document (or file) may contain US data and be controlled for export under the Export Administration
Regulations (EAR), 15 CFR Parts 730-774, ECCN: 9E991. Violations of these laws may be subject to fines and penalties under the Export
Administration Act.

©

SAFRAN LANDING SYSTEMS 2016 (AND SUBSEQUENT REVISION PAGE DATES)

This document and all information contained herein is the sole property of Safran Landing Systems (and/or its affiliated companies).

No intellectual property rights are granted by the delivery of this document or the disclosure of its content. This document shall not be
reproduced to a third party without the express written consent of Safran Landing Systems (and/or the appropriate affiliated company).

TITLE PAGE

32-12-22 Sep 16/2016

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

INTENTIONALLY BLANK

32-12-22

>SAFRAN

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL

MAIN LANDING GEAR LEG

TABLE OF CONTENTS

Page

Description and Operation 	 1

General 	 1

Description	 1

Operation 	 1

Data 	 2

Testing and Fault Isolation 	 101

Equipment and Materials 	 101

Test Conditions 	 102

Procedure 	 102

Fault Isolation	 109

Disassembly 	 301

General 	 301

Procedure 	 307

Cleaning 	 401

General 	 401

Procedure 	 401

Check 	 501

General 	 501

Detailed Inspection 	 501

Special Detailed Inspection 	 501

Repair 	 601

General 	 601

Repair Procedure Conditions	 602

Protective Treatment 	 602

Approved Repairs 	 689

Repair No. 1-1 Lower Bearing	Subassembly	 Repair No.	1-1	601

Repair No. 1-2 Lower Bearing	Subassembly	 Repair No.	1-2	601

Repair No. 2-1 Pin 	Repair	No.	2-1	601

Repair No. 3-1 Pivot Pin 	Repair	No.	3-1	601

Repair No. 4-1 Uplock Pin 	Repair	No.	4-1	601

Repair No. 5-1 Pin 	Repair	No.	5-1	601

Repair No. 6-1 Pin 	Repair	No.	6-1	601

Repair No. 6-2 Pin 	Repair	No.	6-2	601

Repair No. 6-3 Pin 	Repair	No.	6-3	601

Repair No. 7-1 Pin 	Repair	No.	7-1	601

Repair No. 7-2 Pin 	Repair	No.	7-2	601

Repair No. 8-1 Bracket 	Repair	No.	8-1	601

Repair No. 8-2 Bracket 	Repair	No.	8-2	601

Repair No. 9-1 Sliding Tube	Repair No.	9-1	601

Repair No. 9-2 Sliding Tube	Repair No.	9-2	601

Repair No. 9-3 Sliding Tube (Superseded) 	 Repair	No.	9-3	601

Repair No. 9-4 Sliding Tube	Repair No.	9-4	601

Repair No. 9-5 Sliding Tube	Repair No.	9-5	601

TABLE OF CONTENTS

32

Page 1

--	Mar 18/2025

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG
TABLE OF CONTENTS (Continued)

Page

Repair No. 9-6 Sliding Tube 	 Repair	No.	9-6	601

Repair No. 9-7 Sliding Tube 	 Repair	No.	9-7	601

Repair No. 9-8 Sliding Tube 	 Repair	No.	9-8	601

Repair No. 9-9 Sliding Tube 	 Repair	No.	9-9	601

Repair No. 9-10 Sliding Tube 	 Repair	No.	9-10	601

Repair No. 9-11 Sliding Tube 	 Repair	No.	9-11	601

Repair No. 10-1 Pin 	 Repair	No.	10-1	601

Repair No. 11-1 Main Fitting 	 Repair	No.	11-1	601

Repair No. 11-2 Main Fitting 	 Repair	No.	11-2	601

Repair No. 11-3 Main Fitting 	 Repair	No.	11-3	601

Repair No. 11-4 Main Fitting 	 Repair	No.	11-4	601

Repair No. 11-5 Main Fitting 	 Repair	No.	11-5	601

Repair No. 11-6 Main Fitting 	 Repair	No.	11-6	601

Repair No. 11-7 Main Fitting 	 Repair	No.	11-7	601

Repair No. 11-8 Main Fitting 	 Repair	No.	11-8	601

Repair No. 11-9 Main Fitting 	 Repair	No.	11-9	601

Repair No. 11-10 Main Fitting (Withdrawn)	Repair	No.	11-10	601

Repair No. 11-11 Main Fitting 	 Repair	No.	11-11	601

Repair No. 11-12 Main Fitting 	 Repair	No.	11-12	601

Repair No. 11-13 Main Fitting 	 Repair	No.	11-13	601

Repair No. 11-14 Main Fitting	 Repair	No.	11-14	601

Repair No. 11-15 Main Fitting 	 Repair	No.	11-15	601

Repair No. 11-16 Main Fitting 	 Repair	No.	11-16	601

Repair No. 11-17 Main Fitting 	 Repair	No.	11-17	601

Repair No. 11-18 Main Fitting 	 Repair	No.	11-18	601

Repair No. 11-19 Main Fitting	 Repair	No.	11-19	601

Repair No. 11-20 Main Fitting (Withdrawn)	Repair	No.	11-20	601

Repair No. 11-21 Main Fitting 	 Repair	No.	11-21	601

Repair No. 11-22 Main Fitting	 Repair	No.	11-22	601

Repair No. 11-23 Main Fitting 	 Repair	No.	11-23	601

Repair No. 11-24 Main Fitting 	 Repair	No.	11-24	601

Repair No. 11-25 Main Fitting 	 Repair	No.	11-25	601

Repair No. 11-26 Main Fitting 	 Repair	No.	11-26	601

Repair No. 11-27 Main Fitting	 Repair	No.	11-27	601

Repair No. 11-28 Main Fitting 	 Repair	No.	11-28	601

Repair No. 11-29 Main Fitting	 Repair	No.	11-29	601

Repair No. 11-30 Main Fitting 	 Repair	No.	11-30	601

Repair No. 11-31 Main Fitting	 Repair	No.	11-31	601

Repair No. 11-32 Main Fitting	 Repair	No.	11-32	601

Repair No. 11-33 Main Fitting	 Repair	No.	11-33	601

Repair No. 11-34 Main Fitting	 Repair	No.	11-34	601

Repair No. 11-35 Main Fitting	 Repair	No.	11-35	601

Repair No. 11-36 Main Fitting	 Repair	No.	11-36	601

32-12-22

TABLE OF CONTENTS

Page 2

Mar 18/2025

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL

MAIN LANDING GEAR LEG

TABLE OF CONTENTS (Continued)

Page

I Repair No. 11-37 Main Fitting 	 Repair	No. 11-37	601

Repair No.	12-1	Lower Torque Link 	Repair	No.	12-1	601

Repair No.	12-2	Upper Torque Link 	Repair	No.	12-2	601

Repair No.	12-3	Upper Torque Link 	Repair	No.	12-3	601

Repair No.	12-4	Lower Torque Link 	Repair	No.	12-4	601

Repair No.	12-5	Upper Torque Link 	Repair	No.	12-5	601

Repair No.	13-1	Upper Slave Link 	Repair	No.	13-1	601

Repair No.	13-2	Upper Slave Link 	Repair	No.	13-2	601

Repair No.	14-1	Upper Diaphragm Tube 	Repair	No.	14-1	601

Repair No.	14-2	Upper Diaphragm Tube 	Repair	No.	14-2	601

Repair No.	14-3	Upper Diaphragm Tube 	Repair	No.	14-3	601

I Repair No.	14-4	Upper Diaphragm Tube 	Repair	No.	14-4	601

Repair No.	15-1 Bracket 	Repair	No.	15-1	601

Repair No.	15-2 Bracket 	Repair	No.	15-2	601

Repair No.	16-1 Bracket 	Repair	No.	16-1	601

Repair No.	17-1 Pivot Bracket	Repair No. 17-1	601

Repair No.	18-1 Cylinder	Repair No. 18-1	601

Repair No.	18-2 Cylinder	Repair No. 18-2	601

Repair No.	18-3 Cylinder (Withdrawn) 	 Repair	No.	18-3	601

Repair No.	18-4 Cylinder	Repair No. 18-4	601

Repair No.	18-5 Cylinder	Repair No. 18-5	601

Repair No.	18-6 Cylinder	Repair No. 18-6	601

Repair No.	18-7 Cylinder	Repair No. 18-7	601

I Repair No.	18-8 Cylinder	Repair No. 18-8	601

Repair No.	19-1 Lock Stay Cardan 	Repair	No.	19-1	601

Repair No.	19-2 Lock Stay Cardan 	Repair	No.	19-2	601

Repair No.	20-1 Lower Slave Link 	Repair	No.	19-2	601

Repair No.	21-1 Transfer Block 	Repair	No.	21-1	601

Repair No.	21-2 Transfer Block 	Repair	No.	21-2	601

Repair No.	21-3 Transfer Block 	Repair	No.	21-3	601

Repair No.	21-4 Transfer Block	Repair No. 21-4	601

Repair No.	21-5 Transfer Block	Repair No. 21-5	601

Repair No.	21-6 Transfer Block	Repair No. 21-6	601

Repair No.	22-1 Spherical Bearing 	Repair	No.	22-1	601

Repair No.	23-1 Pintle pin	Repair No. 23-1	601

Repair No.	23-2 Pintle Pin	Repair No. 23-2	601

Repair No.	24-1 Harness Support Bracket 	Repair	No.	24-1	601

Repair No.	25-1 Retaining Pin 	Repair	No.	25-1	601

Repair No.	25-2 Retaining Pin 	Repair	No.	25-2	601

Repair No.	26-1 Upper Pivot Bracket 	Repair	No.	26-1	601

Assembly (Including Storage) 	 701

General 	 701

Procedure 	 706

TABLE OF CONTENTS

32-12-22

Page 3

Mar 18/2025

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL

MAIN LANDING GEAR LEG

TABLE OF CONTENTS (Continued)

Page

Storage	798.12

Fits and Clearances 	 801

Fits and Clearances Definitions 	 801

Remarks	 869

Torque Data 	 869

Special Tools, Fixtures and Equipment 	 901

General 	 901

Illustrated Parts List	 1001

Introduction 	 1002

How To Use This Illustrated Parts List 	 1003

Vendor Codes, Names and Addresses 	 1004

Numerical Index	 1005

Detailed Parts List 	 1031

ILLUSTRATIONS

Fig.	Page

- Main Landing Gear Leg 	 3

- Diagram of Operation	 4

101	Main Landing Gear Leg (1-1)	 105

102	Electrical Bonding Resistance Test Points (Tables 101 and	102) 	 108

601	Bracket (2-120 and 2-130) - Protective Treatment	 622

602	Transfer Block (2-340, 2-340A, 2-350 and 2-350A) - Protective	Treatment	 623

603	Bracket (4-140 and 4-150) - Protective Treatment	 624

604	Bracket (5-160) - Protective Treatment 	 625

605	Bracket (6-170 and 6-180) - Protective Treatment	 626

606	Slave Link (6-230) - Protective Treatment	 627

607	Slave Link (6-230A) - Protective Treatment	 628

608	Pin (10-80) - Protective Treatment	 629

609	  630

610	Pin (11-130) - Protective Treatment	 631

611	  632

612	Valve Stem (12-90) - Protective Treatment 	 633

613	Retaining Pin (13-10) - Protective Treatment	 633

614	Inflation Valve (13-110A) - Protective Treatment 	 634

615	Pin (13-190 and 13-190A) - Protective Treatment	 635

616	Upper Diaphragm Tube (15-390) - Protective Treatment	 636

617	Cylinder (17-230) - Protective Treatment	 637

618	Sliding Tube (18-80) or (18-80A) - Protective Treatment	(Sheet	1) 	 638

618	Sliding Tube (18-80) or (18-80A) - Protective Treatment	(Sheet	2) 	 639

619 Main Fitting Subassembly (20-90, 20-90A, 20-100 and 20-100A)

- Protective Treatment (Sheet 1) 	 640

619 Main Fitting Subassembly (20-90, 20-90A, 20-100 and 20-100A)

- Protective Treatment (Sheet 2) 	 641

620 Main Fitting (20-410) and (20-420) - Protective Treatment 	 642

32-12-22

TABLE OF CONTENTS

Page 4

Mar 18/2025

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG
TABLE OF CONTENTS (Continued)

ILLUSTRATIONS (Continued)

Fig.

Page

621	Main Fitting (20-410) and (20-420) - Protective Treatment 	 643

622	Main Fitting (20-410A) and (20-420A) - Protective Treatment	(Sheet	1)	 644

622	Main Fitting (20-410A) and (20-420A) - Protective Treatment	(Sheet	2)	 645

623	Upper Diaphragm Tube (15-390A) - Protective Treatment 	 646

624	Cylinder (17-230A) - Protective Treatment 	 647

625	Sliding Tube (18-80B) -	Protective Treatment (Sheet 1) 	 648

625	Sliding Tube (18-80B) -	Protective Treatment (Sheet 2) 	 649

625	Sliding Tube (18-80B) -	Protective Treatment (Sheet 3) 	 650

625	Sliding Tube (18-80B) -	Protective Treatment (Sheet 4) 	 651

626	Upper Pivot Bracket (10-160) Only - Protective Treatment 	 652

627	Upper Pivot Bracket (10-160A) - Protective Treatment 	 653

628	Valve Stem (12-90A) - Protective Treatment 	 654

629	Upper Diaphragm Tube Subassembly (15-360) - Protective Treatment 	 655

630	Upper Diaphragm Tube Subassembly (15-360A) - Protective Treatment 	 656

631	Bracket (2-80 and 2-90) - Protective Treatment 	 657

632	Harness Support Bracket (7-100, 7-100A, 7-110 and 7-110A)	- Protective Treatment	658

633	Bracket (8-170) - Protective Treatment 	 659

634	Harness Support Bracket (11-140) - Protective Treatment 	 660

635	Transfer block (2-310B and 2-320B) - Protective Treatment -	Sheet 1 	 661

635	Transfer block (2-310B and 2-320B) - Protective Treatment -	Sheet 2 	 662

636 Main Fitting (20-410B, 20-420B, 20-410C, 20-420C, 20-410D and 20-420D) -

Protective Treatment - Sheet 1 	 663

636 Main Fitting (20-410B, 20-420B, 20-410C, 20-420C, 20-410D and 20-420D) -

Protective Treatment - Sheet 2 	 664

636 Main Fitting (20-410B, 20-420B, 20-410C, 20-420C, 20-410D and 20-420D) -

Protective Treatment - Sheet 3 	 665

636 Main Fitting (20-410B, 20-420B, 20-410C, 20-420C, 20-410D and 20-420D) -

Protective Treatment - Sheet 4 	 666

636 Main Fitting (20-410B, 20-420B, 20-410C, 20-420C, 20-410D and 20-420D) -

Protective Treatment - Sheet 5 	 667

636 Main Fitting (20-410B, 20-420B, 20-410D and 20-420D) - 	
Protective Treatment - Sheet 6 	 668

636

637

638

638

639

640

641

642

643

644

645

Main Fitting (20-410B, 20-420B, 20-410D and 20-420D) -

Protective Treatment - Sheet 7 	 669

Uplock Pin ain Fitting (5-400A) - Protective Treatment 	 670

Main Fitting (20-90B) and (20-100B) - Protective Treatment - Sheet 1 	 671

Main Fitting (20-90B) and (20-100B) - Protective Treatment - Sheet 2	 672

Forward Pintle Pin (1-60A) - Protective Treatment 	 673

Spacer (4-180) Only - Protective Treatment 	 674

Drag-arm Spacer (4-180A) - Protective Treatment 	 674

Bracket (4-210) - Protective Treatment 	 675

Bracket (9-150) - Protective Treatment 	 676

Bracket (9-150A) - Protective Treatment 	 677

Spacer (9-190) - Protective Treatment 	 678

TABLE OF CONTENTS

32-12-22

Page 5

Mar 18/2025

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG
TABLE OF CONTENTS (Continued)

ILLUSTRATIONS (Continued)

Fig.

Page

646

647

648

648

648

648

648

648

648

648

649

650

651

652

653

654

655

656

657

601

601

601

601

601

601

601

601

601

601

601

601

Spacer (9-190A) - Protective Treatment 	 679

Spacer (11-30) - Protective Treatment	 680

Sliding tube (18-80D, 18-80E, 18-80F and 18-80G) -

Protective Treatment - Sheet 1 	 681

Sliding tube (18-80D, 18-80E, 18-80F and 18-80G) -

Protective Treatment - Sheet 2 	 682

Sliding tube (18-80D, 18-80E, 18-80F and 18-80G) -

Protective Treatment - Sheet 3 	 683

Sliding tube (18-80D, 18-80E, 18-80F and 18-80G) -

Protective Treatment - Sheet 4 	 684

Sliding tube (18-80D, 18-80E, 18-80F and 18-80G) -

Protective Treatment - Sheet 5 	 685

Sliding tube (18-80D, 18-80E, 18-80F and 18-80G) -

Protective Treatment - Sheet 6 	 686

Sliding tube (18-80D, 18-80E, 18-80F and 18-80G) -

Protective Treatment - Sheet 7 	 687

Sliding tube (18-80D, 18-80E, 18-80F and 18-80G) -

Protective Treatment - Sheet 8 	 688

Approved Repairs - Key Diagram 	 693

Main Fitting Repairs - Key Diagram 	 694

Torque Link Repairs - Key Diagram 	 695

Sliding Tube Repairs - Key Diagram 	 696

Upper Diaphragm Tube Repairs - Key Diagram 	 697

Cylinder Repairs - Key Diagram 	 698

Transfer Block Repairs - Key Diagram	 698.1

Harness Support Bracket Repairs - Key Diagram 	 698.2

Upper Pivot Bracket Repairs - Key Diagram 	 698.2

Repair to Lower Bearing Subassembly 	Repair	No.	1-1	604

Repair to Lower Bearing Subassembly 	Repair	No.	1-2	604

Repair to Pin 	Repair	No.	2-1	602

Repair to Pivot Pin 	Repair	No.	3-1	603

Repair to Uplock Pin 	Repair	No.	4-1	602

Repair to Pin 	Repair	No.	5-1	602

Repair to Pin 	Repair	No.	6-1	603

Repair to Pin 	Repair	No.	6-2	603

Repair to Pin 	Repair	No.	6-3	603

Repair to Pin 	Repair	No.	7-1	603

Repair to Pin 	Repair	No.	7-2	603

Repair to Bracket 	Repair	No.	8-1	603

32-12-22

TABLE OF CONTENTS

Page 6

Mar 18/2025

^SAFRAN

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

TABLE OF CONTENTS (Continued)
ILLUSTRATIONS (Continued)

Fig.	Page

601 Repair to Bracket	Repair	No. 8-2	603

601	Repair to	Sliding	Tube 	 Repair	No.	9-1	603

602	Repair to	Sliding	Tube 	 Repair	No.	9-1	604

601	Repair to	Sliding	Tube 	 Repair	No.	9-2	604

601	Repair to	Sliding	Tube -	Machining 	 Repair	No.	9-4	603

602	Repair Bush - Machining	and Installation 	 Repair	No.	9-4	604

601	Repair to	Sliding	Tube 	 Repair	No.	9-5	602

601	Repair to	Sliding	Tube 	 Repair	No.	9-6	603

601	Repair to Sliding	Tube - Machining 	 Repair	No.	9-7	603

602	Repair Bushes -	Machining and Installation 	 Repair	No.	9-7	604

601	Repair to Sliding	Tube - Machining 	 Repair	No.	9-8	604

602	Repair Bushes -	Machining and Installation 	 Repair	No.	9-8	605

601	Repair to Sliding	Tube - Machining 	 Repair	No.	9-9	603

602	Repair Bushes -	Machining and Installation 	 Repair	No.	9-9	604

601	Repair to	Sliding	Tube 	Repair	No.	9-10	605

602	Repair to	Sliding	Tube 	Repair	No.	9-10	606

601	Repair to	Sliding	Tube 	 Repair	No.	9-11	603

602	Repair to	Sliding	Tube -	Machining 	 Repair	No.	9-11	604

603	Repair Bushes -	Machining	and	Installation 	 Repair	No.	9-11	605

601	Repair to	Pin 	Repair	No.	10-1	603

601	Repair to	Main	Fitting	 Repair No.	11-1	603

602	Oversize	Bushes - Machining and Installation	 Repair No.	11-1	604

601	Repair to	Main	Fitting	 Repair No.	11-2	603

602	Oversize Bush(es) - Machining and Installation 	 Repair	No.	11-2	604

601	Repair to Main Fitting	 Repair No.	11-3	604

602	Repair to Main Fitting	 Repair No.	11-3	605

603	Repair to Main Fitting - Repair Bush	Machining and	Installation 	 Repair	No.	11-3	606

601	Repair to Main Fitting	 Repair No.	11-4	603

602	Repair Bush - Machining	and Installation 	 Repair	No.	11-4	604

601	Repair to Main Fitting	 Repair No.	11-5	603

602	Repair Bush - Machining	and Installation 	 Repair	No.	11-5	604

601	Repair to Main Fitting	 Repair No.	11-6	603

602	Repair Bush - Machining	and Installation 	 Repair	No.	11-6	604

601	Repair to Main Fitting 	 Repair	No.	11-7	603

602	Repair to Main Fitting 	 Repair	No.	11-7	604

603	Repair to Main Fitting - Repair Bush	Machining and	Installation	 Repair No.	11-7	605

601	Repair to Main Fitting 	 Repair	No.	11-8	603

602	Repair to Main Fitting - Repair Bush	Machining and	Installation	 Repair No.	11-8	604

601	Repair to Main Fitting 	 Repair	No.	11-9	604

602	Repair to Main Fitting - Machining 	 Repair	No.	11-9	605

603	Repair to Main Fitting - Repair Bush	Installation 	 Repair	No.	11-9	606

601	Repair to Main Fitting - Machining 	 Repair	No.	11-11	603

602	Repair Bearing - Machining and Installation 	 Repair	No.	11-11	604

601	Repair to Main Fitting - Machining 	 Repair	No.	11-12	603

602	Repair Bush - Machining and Installation 	 Repair	No.	11-12	604

601	Repair to Main Fitting - Machining 	 Repair	No.	11-13	604

TABLE OF CONTENTS

32

Page 7

--	Mar 18/2025

^SAFRAN

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

TABLE OF CONTENTS (Continued)
ILLUSTRATIONS (Continued)

Fig.	Page

602	Repair to Bushes - Machining and	Installation	 Repair	No. 11-13	605

601	Repair to Main Fitting - Machining		 Repair	No. 11-14	603

601	Repair to Main	Fitting	 Repair	No. 11-15	603

602	Repair to Main	Fitting	- Machining 	 Repair	No. 11-15	604

603 Repair Bush - Machining and installation	 Repair	No. 11-15	605

601	Repair to Main	Fitting	Repair	No. 11-16	602

601	Repair to Main	Fitting	Repair	No. 11-17	602

601	Repair to Main	Fitting	- Machining 	 Repair	No. 11-18	603

602	Repair Bearing	- Machining and Installation 	 Repair	No. 11-18	604

601	Repair to Main	Fitting	- Machining 	 Repair	No. 11-19	603

602	Repair Bushes	- Machining and Installation	 Repair	No. 11-19	604

601	Repair to Main	Fitting	- Machining 	 Repair	No. 11-21	603

602	Repair Bush - Machining and Installation 	 Repair	No. 11-21	604

601	Repair to Main Fitting	Repair	No. 11-22	602

601	Repair to Main Fitting	Repair	No. 11-23	603

602	Repair to Main Fitting - Machining		 Repair	No. 11-23	604

603	Oversize Lubrication adapter - Installation	 Repair	No. 11-23	605

601	Repair to Main	Fitting	Repair	No. 11-24	603

602	Repair to Main	Fitting	Repair	No. 11-24	604

601	Repair to Main	Fitting	Repair	No. 11-25	603

601	Repair to Main	Fitting	Repair	No. 11-26	604

602	Repair to Main	Fitting	- Machining 	 Repair	No. 11-26	605

603	Lower Bearing Subassembly Machining and Liner Installation 	 Repair	No. 11-26	606

601	Repair to Main	Fitting	Repair	No. 11-27	603

602	Repair Bearing - Machining and Installation 	 Repair	No. 11-27	604

601	Repair to Main	Fitting	Repair	No. 11-28	604

602	Repair to Main	Fitting		 Repair	No. 11-28	605

603	Oversize Transfer Dowel - Installation	 Repair	No. 11-28	606

601	Repair to Main Fitting - Machining	 Repair	No. 11-29	606

602	Repair to Main Fitting 	 Repair	No. 11-29	607

603	Repair to Main Fitting - Chromium	Plate	Termination 	 Repair	No. 11-29	608

601	Repair to Main Fitting - Machining		 Repair	No. 11-30	605

602	Repair to Main Fitting	Repair	No. 11-30	606

603	Repair to Main Fitting - Chromium	Plate	Termination 	 Repair	No. 11-30	607

604 Lower Bearing Subassembly - Machining and Inner Liner InstallationRepair No. 11-30	608

601	Repair to Main Fitting 	 Repair	No. 11-31	604

601	Repair to Main	Fitting	- Sheet 1 	 Repair	No. 11-32	604

601	Repair to Main	Fitting	- Sheet 2	Repair	No. 11-32	605

601	Repair to Main	Fitting	- Sheet 3	Repair	No. 11-32	606

601	Repair to Main	Fitting	- Sheet 1 	Repair	No. 11-33	604

601	Repair to Main	Fitting	- Sheet 2	Repair	No. 11-33	605

601	Repair to Main	Fitting	- Sheet 3	Repair	No. 11-33	606

TABLE OF CONTENTS

32

Page 8

--	Mar 18/2025

♦ SAFRAN

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

TABLE OF CONTENTS (Continued)
ILLUSTRATIONS (Continued)

Fig.	Page

601	Repair to Main	Fitting	- Sheet	1 	 Repair	No.	11-34	603

601	Repair to Main	Fitting	- Sheet	2	 Repair	No.	11-34	604

602	Repair to Main	Fitting	- Bush 	 Repair	No.	11-34	605

601	Repair to Main	Fitting	- Sheet	1 	 Repair	No.	11-35	603

601	Repair to Main	Fitting	- Sheet	2	 Repair	No.	11-35	604

602	Repair to Main	Fitting	- Bush 	 Repair	No.	11-35	605

601	Repair to Main	Fitting	- Sheet	1 	 Repair	No.	11-36	603

601	Repair to Main	Fitting	- Sheet	2	 Repair	No.	11-36	604

601	Repair to Main	Fitting	- Sheet	1 	 Repair	No.	11-37	603

601	Repair to Main	Fitting	- Sheet	2	 Repair	No.	11-37	604

601	Repair to Main	Fitting	- Sheet	3	 Repair	No.	11-37	605

602 Repair sleeve 	 Repair	No. 11-37	606

601	Repair to Lower Torque Link - Machining 	 Repair	No.	12-1	603

602	Repair Bushes - Machining and Installation 	Repair	No.	12-1	604

601	Repair to Upper Torque Link - Machining 	 Repair	No.	12-2	604

602	Repair Bushes - Machining and Installation 	Repair	No.	12-2	605

603	Repair Sleeves - Machining and Installation 	Repair	No.	12-2	606

601	Repair to Upper Torque Link - Machining 	 Repair	No.	12-3	603

602	Repair Bushes - Machining and Installation 	Repair	No.	12-3	604

601	Repair to Lower Torque Link - Machining 	 Repair	No.	12-4	603

602	Repair Bushes - Machining and Installation 	 Repair	No.	12-4	604

601	Repair to Upper Torque Link - Machining 	 Repair	No.	12-5	603

602	Repair Sleeves - Machining and Installation 	 Repair	No.	12-5	604

601	Repair to	Upper Slave	Link - Machining 	 Repair	No.	13-1	604

602	Repair to	Upper Slave	Link - Machining 	 Repair	No.	13-1	605

603	Repair Bushes - Machining and Installation 	 Repair	No.	13-1	606

601	Repair to	Upper Slave	Link - Machining 	 Repair	No.	13-2	603

602	Repair Bushes - Installation 	 Repair	No.	13-2	604

601	Repair to Upper Diaphragm Tube - Machining 	 Repair	No.	14-1	603

602	Repair Bush - Machining and Installation 	 Repair	No.	14-1	604

601	Repair to Upper Diaphragm Tube -	Machining 	 Repair	No.	14-2	603

602	Repair Bush - Machining and Installation 	 Repair	No.	14-2	604

601	Repair to Upper Diaphragm Tube 	 Repair	No.	14-3	604

602	Repair to Upper Diaphragm Tube 	 Repair	No.	14-3	605

601	Repair to Upper Diaphragm Tube -	Sheet 1 	Repair	No.	14-4	605

601	Repair to Upper Diaphragm Tube -	Sheet 2 	Repair	No.	14-4	606

601	Repair to Bracket - Machining	 Repair	No.	15-1	603

602	Repair Bearing - Machining and Installation 	 Repair	No.	15-1	604

601	Repair to Bracket - Machining	 Repair	No.	15-2	603

602	Repair Bearing - Machining and Installation 	 Repair	No.	15-2	604

601	Repair to Bracket - Machining	 Repair	No.	16-1	603

602	Repair Bushes - Machining and Installation 	 Repair	No.	16-1	604

TABLE OF CONTENTS

32

Page 9

--	Mar 18/2025

^SAFRAN

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

TABLE OF CONTENTS (Continued)
ILLUSTRATIONS (Continued)

Fig.	Page

601	Repair to	Pivot Bracket 	 Repair	No.	17-1	603

601	Repair to	Cylinder 	 Repair	No.	18-1	602

601	Repair to	Cylinder - Machining 	 Repair	No.	18-2	603

602 Oversize Thread Insert - Installation 	 Repair	No. 18-2	604

601	Repair to	Cylinder 	 Repair	No.	18-4	602

601	Repair to	Cylinder 	 Repair	No.	18-5	603

601	Repair to	Cylinder 	 Repair	No.	18-6	603

601	Repair to	Cylinder 	 Repair	No.	18-7	602

601	Repair to	Cylinder 	Repair	No.	18-8	603

602	Repair to	Cylinder 	Repair	No.	18-8	604

601	Repair to	Lock Stay Cardan 	 Repair	No.	19-1	603

602	Repair to	Lock Stay Cardan - Machining	 Repair	No.	19-1	604

603	Repair Bushes - Machining and	Installation	Repair No.	19-1	605

601	Repair to Lock Stay Cardan - Machining 	Repair	No.	19-2	603

602	Repair Bushes - Machining and	Installation 	Repair	No.	19-2	604

601	Repair to Lower Slave Link 	Repair	No.	20-1	603

601	Repair to Transfer Block - Machining	Repair No. 21-1	603

602	Repair Bush - Machining and Installation 	Repair	No.	21-1	604

601	Repair to Transfer Block - Machining	Repair No. 21-2	603

602	Repair Bush - Machining and Installation 	Repair	No.	21-2	604

601	Repair to Transfer Block - Machining	Repair No. 21-3	603

602	Repair Threaded Insert - Machining and Installation 	Repair	No.	21-3	604

601	Repair to Transfer Block - Machining	Repair	No.	21-4	603

602	Repair Bush - Machining and Installation 	Repair	No.	21-4	604

601	Repair to Transfer Block - Machining 	 Repair	No.	21-5	603

602	Repair Bush - Machining and Installation 	 Repair	No.	21-5	604

601	Repair to Transfer Block - Machining 	 Repair	No.	21-6	603

602	Repair Threaded Insert - Machining and Installation	 Repair	No.	21-6	604

601	Repair to Spherical Bearing	 Repair	No.	22-1	602

601	Repair to Pintle Pin 	 Repair	No.	23-1	603

601	Repair to Pintle Pin 	 Repair	No.	23-2	603

602	Repair Bushes - Machining and Installation 	 Repair	No.	23-2	604

601	Repair to Harness Support Bracket	 Repair	No.	24-1	603

602	Repair Bushes - Machining and Installation 	 Repair	No.	24-1	604

601	Repair to Retaining Pin 	 Repair	No.	25-1	603

601	Repair to Retaining Pin 	 Repair	No.	25-2	603

601	Repair to Upper Pivot Bracket - Machining 	 Repair	No.	26-1	603

602	Repair Sleeve - Machining and Installation 	 Repair	No.	26-1	604

701	Main Fitting Subassembly (20-90) - Bearing (20-300) - Installation (Sheet 1 of 2) 	 711

701	Main Fitting Subassembly (20-90) - Ardrox Application (Sheet 2 of 2) 	 712

702 Labels (20-10, 20-30, 20-40, 20-60 and 20-80) and wiring diagram plate (1-110)

- Installation 	 713

TABLE OF CONTENTS

32

Page 10

--	Mar 18/2025

^SAFRAN

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG
TABLE OF CONTENTS (Continued)

ILLUSTRATIONS (Continued)

Fig.	Page

703	Main Fitting	Subassembly (20-90B) or	(20-90C)	- Installation	(Sheet 1 of	6) 	 722

703	Main Fitting	Subassembly (20-90B) or	(20-90C)	- Installation	(Sheet 2 of	6) 	 723

703	Main Fitting	Subassembly (20-90B) or	(20-90C)	- Installation	(Sheet 3 of	6) 	 724

703	Main Fitting	Subassembly (20-90B) or	(20-90C)	- Installation	(Sheet 4 of	6) 	 725

703	Main Fitting	Subassembly (20-90B) or	(20-90C)	- Installation	(Sheet 5 of	6) 	 726

703	Main Fitting	Subassembly (20-90B) or	(20-90C)	- Installation	(Sheet 6 of	6) 	 727

704	Installation of Labels 	 728

705	Label (18-70) - Installation	 731

706	Label (17-290) Only and Bush (18-50) Only - Installation	 732

707	Sliding Tube Subassembly	(17-240B	or 17-240G) -	Installation of Bushes	(Sheet 1	of	5)	737

707	Sliding Tube Subassembly	(17-240B	or 17-240G) -	Installation of Bushes	(Sheet 2	of	5)	738

707 Sliding Tube Subassembly (17-240B or 17-240C or 17-240F) -
Installation of Bushes (Sheet 3 of 5) 	 739

707	Sliding Tube Subassembly (17-240C	or	17-240F) -	Installation	of Bushes (Sheet	4	of 5)	740

707	Sliding Tube Subassembly (17-240C	or	17-240F) -	Installation	of Bushes (Sheet	5	of 5)	741

708	Bushes (18-20 and 18-30) - Installation	 743

709	Label (17-250) - Installation	 743

710	Application of Jointing Compound 	 751

711 Pre SB 201-32-49 or Pre SB 201-32-58 or Pre SB 201-32-60 Lower Bearing (16-150)

or (16A-150) Grease Groove Dimensions After Installation in the Gland Housing (16-140)

or (16A-140) or (16A-140A)	 752

712 Post SB-201-32-58 - Lower Bearing Subassembly Machining and Liner Installation .
(Sheet 1 of 2)	 757

712	Assembly of Lower Bearing Subassembly (16A-110C) (Sheet	2	of 2) 	 758

713	Assembly of Lower Bearing Subassembly (16-110D or 16A-110E)	 759

714	Seal Configuration (Sheet 1 of 2) 	 760

714	Seal Configuration (Sheet 2 of 2) 	 761

715 Upper Diaphragm Tube Subassembly (15-360 only)	 768

716 Crimping of the Pin (15-120) 	 769

717 Upper Diaphragm Tube Subassembly (15-360A) 	 776

718	Locking Plate (15-80) - Hole Locations 	 777

719	Application of Ardrox AV100D to	the Upper Diaphragm	Tube	(15-390) (Sheet	1	of 3)	778

719	Application of Ardrox AV100D to	the Upper Diaphragm	Tube	(15-390A)	(Sheet	2 of	3)	779

719	Application of Ardrox AV100D to	the Pin (13-190) (Sheet 3 of	3) 	 780

720 Bushes (10-240, 10-250, 11-220 and 11-230) - Installation	 785

721 1M Electrical Axle Harness (11-40) and 2M Electrical Axle Harness (11-50)- 	
Installation 	 787

722	Assembly of Damper (9-160)	 789

723	Bracket Subassembly (5-270) - Installation 	 797

724	Bracket Subassembly (5-90) - Installation 	 798

725	Bushes (3-150 and 3-160) - Installation	 798.3

726	Bolt Subassembly (1-40A) - Assembly 	 798.7

TABLE OF CONTENTS

32

Page 11

--	Mar 18/2025

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG
TABLE OF CONTENTS (Continued)

ILLUSTRATIONS (Continued)

Fig.

Page

727

728

801

802

803

804

805

806

807

808

809

810

811

812

813

814

815

816

817

818

819

820

821

822

823

824

IPL 1

IPL 2

IPL 2

IPL 3

IPL 4

IPL 5

IPL 6

IPL 7

IPL 8

IPL 9

PL 10

IPL 11

IPL 12

IPL 13

IPL 14

IPL 15

IPL 16

Proximity Switch (7-230) and Target (7-180) - Adjustment	 798.11

Proximity Switch (7-40) and Target (6-130) - Adjustment	 798.11

Fits and Clearances - Key Diagram	 802

Fits and Clearances - Key Diagram	 803

Fits and Clearances (Table 801) 	 804

Fits and Clearances (Table 802) 	 807

Fits and Clearances (Table 803) 	 810

Fits and Clearances (Table 804) 	 814

Fits and Clearances (Table 805) 	 817

Fits and Clearances (Table 806) 	 819

Fits and Clearances (Table 807) 	 821

Fits and Clearances (Table 808) 	 823

Fits and Clearances (Table 809) 	 826

Fits and Clearances (Table 810) 	 828

Fits and Clearances (Table 811) 	 830

Fits and Clearances (Table 812) 	 832

Fits and Clearances (Table 813) 	 836

Fits and Clearances (Table 814) 	 840

Fits and Clearances (Table 815) 	 845

Fits and Clearances (Table 816) 	 849

Fits and Clearances (Table 817) 	 852

Fits and Clearances (Table 818) 	 855

Fits and Clearances (Table 819) 	 858

Fits and Clearances (Table 820) 	 860

Fits and Clearances (Table 821) 	 864

Fits and Clearances (Table 822) 	 867

Main Landing Gear Leg 	 1032

Main Landing Gear Leg (Sheet 1) 	 1036

Main Landing Gear Leg (Sheet 2) 	 1038

Main Landing Gear Leg 	 1044

Main Landing Gear Leg 	 1048

Main Landing Gear Leg 	 1054

Main Landing Gear Leg 	 1060

Main Landing Gear Leg 	 1064

Main Landing Gear Leg 	 1070

Main Landing Gear Leg 	 1074

Main Landing Gear Leg 	 1078

Main Landing Gear Leg 	 1082

Main Landing Gear Leg 	 1086

Main Landing Gear Leg 	 1090

Main Landing Gear Leg 	 1098

Main Landing Gear Leg 	 1098.4

Main Landing Gear Leg (Pre SB-201-32-58) (Post SB-201-32-78)	 1098.12

32-12-22

TABLE OF CONTENTS

Page 12

Mar 18/2025

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG
TABLE OF CONTENTS (Continued)

ILLUSTRATIONS (Continued)

Fig.

Page

IPL 16AMain Landing	Gear Leg (Post SB-201-32-58) (Pre SB-201-32-78)	 1098.18

IPL 17	Main Landing	Gear Leg	 1098.26

IPL 18 Sliding Tube Subassembly 	 1098.34

IPL 19	Main Landing	Gear Leg	 1098.38

IPL 20	Main Landing	Gear Leg 	 1098.42

TABLE OF CONTENTS

32-12-22

Page 13

Mar 18/2025

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

INTENTIONALLY BLANK

32-12-22

Page 14

TABLE OF CONTENTS

^SAFRAN	SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

INTRODUCTION

- General

- AECMA Simplified English to PSC-85-16598 is used in this manual.

- This manual contains Description, Operation, Maintenance procedures and an Illustrated
Parts List (IPL). IPL Figure and Item numbers in parentheses follow the part name to identify
them.

- A Unit Identification Chart is included to show the modification status of the unit. The
modification status is related to the unit part number by the dash number: the dash number is
marked on the unit name plate adjacent to the part number.

- All references in this manual are to the left configuration of the unit unless the instructions tell
you differently.

- All dimensions and quantities in this manual are in SI units with Imperial units in parentheses.
A comma shows a decimal part of an SI unit. A full point shows a decimal part of an Imperial
unit.

- This manual refers to Process Specifications (M-DLPS and PCS) and Non-destructive Tests
(M-DLNDT). These are available within the Safran Landing Systems Technical Publications
on-line service.

- All the materials in this manual have a Ref. Item identification. This is the reference item
number of the material in the Aircraft Manufacturer’s Consumable Materials List.

- Use approved persons and good aircraft engineering practice for all procedures in this
manual.

- The repairs in this CMM have been approved under Airbus’ EASA Design Organisation
Approval No. EASA.21J.031.

- On occasion a REF. CODE can be identified in the NOMENCLATURE column in the
DETAILED PARTS LIST. This is a Safran Landing Systems reference code and is used for
cross-reference purposes only.

- The accuracy and the adequacy of the instructions in this CMM have been technically verified
by shop verification (performed or simulated) or by similarity with manufacturing instructions
or with component maintenance manuals instructions from other programs that have been
verified in shop.

- Reference Publications

- Safran Landing Systems UK Ltd Component Maintenance Manual, Main Landing Gear Leg
and Dressings, 32-12-21.

- Safran Landing Systems UK Ltd Component Maintenance Manual, Axle Harness 1M and 2M,
32-12-29.

- Safran Landing Systems UK Ltd Component Maintenance Manual, Damper, 32-11-93.

- Safran Landing Systems UK Ltd Component Maintenance Manual, Damper, 32-12-85.

INTRODUCTION

Page 1

Mar 18/2022

32-12-22

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

INTENTIONALLY BLANK

32-12-22

INTRODUCTION

Page 2

^SAFRAN	SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

DESCRIPTION AND OPERATION

- General

A. The main landing gear leg is a two stage, telescopic shock absorber.

- Description (Refer to Figures 1 and 2)

- The main landing gear leg has a sliding tube subassembly that operates in a main fitting
subassembly. The sliding tube subassembly operates through a lower bearing subassembly.
The lower bearing subassembly also seals the sliding tube subassembly in the main fitting
subassembly.

- An upper torque link subassembly attaches to the main fitting subassembly. A lower torque
link subassembly attaches to the sliding tube subassembly. A damper attaches to the upper
torque link subassembly. A pin installs through the damper and connects the upper and lower
torque link subassemblies.

- A slave link subassembly and a lower slave link subassembly attach opposite the upper and
lower torque link subassemblies.

- A rod and a cylinder install in the sliding tube subassembly. A piston installs in the cylinder.
An upper diaphragm tube subassembly installs in the main fitting subassembly. A baffle, a
compression orifice plate and a diaphragm subassembly install in the upper diaphragm tube
subassembly. The rod goes through the baffle.

- An upper bearing housing installs between the top of the sliding tube subassembly and the
main fitting subassembly. A recoil orifice plate operates in the upper bearing housing.

- Operation (Refer to Figure 2)

- Compression

- The sliding tube subassembly moves into the main fitting subassembly. The subsequent
decrease in volume causes hydraulic fluid to flow through the upper bearing housing:
the recoil orifice plate moves and slows the flow of hydraulic fluid. The decrease in
volume also causes hydraulic fluid to move through the diaphragm and lift the
compression orifice plate: the hydraulic fluid flows through the baffle and into the upper
diaphragm tube subassembly. This slows the speed of the compression.

- Hydraulic fluid that moves into the upper diaphragm tube compresses the nitrogen in the
main fitting subassembly and the upper diaphragm tube subassembly. As the pressure
of the nitrogen increases, the hydraulic fluid in the rod moves against the piston. The
piston is pushed into the cylinder and compresses the nitrogen in it. This slows the
speed of the compression more.

- Recoil

- After compression, the nitrogen pressure in the cylinder pushes the piston to the end of
the cylinder: hydraulic fluid moves out of the cylinder and into the rod. The nitrogen
pressure in the main fitting subassembly and the upper diaphragm subassembly pushes
the hydraulic fluid through the baffle: the compression orifice plate is pushed against the
diaphragm subassembly and limits the flow of hydraulic fluid through it. This slows the
speed of the recoil. The sliding tube subassembly moves out of the main fitting
subassembly.

32-12-22

Page 1

Dec 6/2019

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL

MAIN LANDING GEAR LEG

- The Upper and Lower Torque Link Subassemblies

- The upper and lower torque link subassemblies prevent the sliding tube subassembly
from turning in the main fitting subassembly.

- The damper controls the movement of the upper and lower torque link subassemblies.

- Data

522 kg (1151 lb) approximately

505 kg (1113 lb) approximately
	Material Ref. Item 02-501

Weight with hydraulic fluid

Weight without hydraulic fluid

Hydraulic fluid

32-12-22

Page 2

Dec 6/2019

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

SLIDING TUBE
SUBASSEMBLY

MAIN FITTING
SUBASSEMBLY

SLAVE LINK
SUBASSEMBLY

UPPER TORQUE LINK
SUBASSEMBLY

DAMPER

LOWER TORQUE LINK
SUBASSEMBLY

LOWER SLAVE LINK
SUBASSEMBLY

A3219168-1

Main Landing Gear Leg
Figure 1

32-12-22

Page 3

Dec 6/2019

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

Diagram of Operation
Figure 2

32-12-22

Page 4

Dec 6/2019

sSAFRAN	SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG
TESTING AND FAULT ISOLATION

- Equipment and Materials

- Equipment

- This equipment is necessary:

NOTE: Alternative equivalents are permitted.

| Part No. | Equipment | Function |
| --- | --- | --- |
| - | Hydraulic Test Rig |  |
| - | Nitrogen Supply | Main landing gear leg (1-1) tests |
| - | Loading Press |  |
|  | Milliohmmeter Megger, | Electrical bonding resistance |
|  | Type BT51 | tests |
| - | 28 VDC Power Supply | Proximity switch and target tests |
| T14218 | Turner Inflation Equipment | Main landing gear leg (1-1) tests |
| T14500 | Crowfoot Wrench | Close the charging valves (13-60 and 17-20) |
| 460002502 | Charging Adapter | Main landing gear leg (1-1) tests |
| 460005842 | Lampbox | Proximity switch and target tests |
| 460006231 | Holding Fixture |  |
| 460006232 | Load Cell and Adapter |  |
| 460006233 | Press Adapter | Main landing gear leg (1-1) tests |
| 460006234 | Offset Adapter |  |
| 460007260 | Bottom Press Adapter |  |

- Materials

- These materials are necessary:

NOTE: Alternative equivalents are permitted.

| Ref. Item | Material |
| --- | --- |
| TBA | Nitrogen |
| 02-501 | Hydraulic fluid |

32-12-22

Page 101

Dec 6/2019

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

- Test Conditions

WARNING: DO NOT GET HYDRAULIC FLUID ON YOUR SKIN OR IN YOUR EYES. DO NOT
BREATHE THE FUMES. ONLY USE IN A LOCATION THAT HAS A CONTINUOUS
FLOW OF CLEAN AIR. HYDRAULIC FLUID IS POISONOUS AND DANGEROUS.

A. General

- The hydraulic test rig must have a hand pump and a power pump. The power pump
must have a controlled flow of not less than 4,5 l/min (4.62 in3/sec).

- The inflation equipment must be to MIL-G-8348.

- The temperature of the test fluid must be between 20 and 40 °C (68 and 104 °F).

- The test fluid must be clean: refer to M-DLPS910-1.

- During all hydraulic tests, the unit and the test circuit must be hydraulically full.

- Examine the unit for damage before you start the tests.

- During the proximity switch tests the ambient temperature must be between 15 and
25 °C (59 and 77 °F).

- Procedure

- Piston (17-200) Leakage Tests

- Use the Charging Adapter 460002502 and the Turner Inflation Equipment T14218:
connect the charging valve (17-20) to the nitrogen supply. Open the charging valve
(17-20).

- Slowly increase the nitrogen pressure to between 9,32 and 10,68 bar (135 and
155 lbf/in2). Make a record of the pressure. Close the charging valve (17-20) and hold
the nitrogen pressure for 15 minutes.

- Open the charging valve (17-20) and measure the nitrogen pressure: it must be the
same as the record in para (2). Leakage must not occur.

- Release the nitrogen pressure.

- Disconnect the nitrogen supply and remove the Turner Inflation Equipment T14218 and
the Charging Adapter 460002502.

- Make sure that all of the nitrogen pressure has been released: remove the charging
valve (17-20).

- Refer to ASSEMBLY: install the charging valve (17-20) and complete the assembly
procedure.

- Main Landing Gear Leg (1-1) Tests

- Initial Operations

- Use these special tools to install the main landing gear leg (1-1) vertically in the
loading press:

1	The Holding Fixture 460006231.

2	The Press Adapter 460006233.

32-12-22

Page 102

Dec 6/2019

sSAFRAN	SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

3 The Bottom Press Adapter 460007260.

- Assemble the Load Cell and Adapter 460006232 and the Offset Adapter
460006234 to the main landing gear leg (1-1).

- Procedure to Fill and Pressurize the Main Landing Gear Leg (1-1)

CAUTION: DO NOT PUT AN END LOAD OF MORE THAN 5,08 TONNES (5 TONS)
ON THE MAIN LANDING GEAR LEG (1-1).

- Make sure that there is no pressure in the main landing gear leg (1-1): open the
charging valves (13-60 and 17-20).

- Use the Charging Adapter 460002502 to connect the hydraulic test rig to the
charging valve (13-60).

- Slowly increase the hydraulic pressure to between 13,11 and 14,48 bar (190 and
210 lbf/in2) and let the unit extend fully.

- Release the hydraulic pressure and fully close the unit.

- Do para (c) and (d) until the hydraulic fluid that comes out of the unit does not have
air in it.

- Fully close the unit and disconnect the hydraulic test rig.

- Use the Charging Adapter 460002502 and the Turner Inflation Equipment T14218
to connect the nitrogen supply to the charging valve (13-60).

CAUTION: DO NOT USE A PRESSURE OF MORE THAN 7,58 BAR (110 LBF/IN2).

- Slowly increase the nitrogen pressure until the unit starts to extend. Hold the
pressure and fully extend the unit. The pressure must not be more than 7,58 bar
(110 lbf/in2).

NOTE: The charging valve (17-20) must be open to let the unit extend fully.

- Refer to Figure 101 and measure the dimension X: it must be between 483,05 and
487,85 mm (19.017 and 19.207 in).

- Disconnect the nitrogen supply and remove the Turner Inflation Equipment T14218
and the Charging Adapter 460002502.

- Open the charging valve (13-60) and release the nitrogen pressure. Do not close
the charging valve (13-60).

- Use the Charging Adapter 460002502 and the Turner Inflation Equipment T14218
to connect the charging valve (17-20) to the nitrogen supply.

- Slowly increase the nitrogen pressure to between 13,11 and 14,48 bar (190 and
210 lbf/in2).

NOTE: Nitrogen will be released through the charging valve (13-60) as the
piston (17-200) moves.

- Slowly increase the nitrogen pressure to between 67,59 and 70,34 bar (980
and1020 lbf/in2).

32-12-22

Page 103

Dec 6/2019

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

- Close the charging valve (17-20); use the Crowfoot Wrench T14500 to torque it to
between 5,7 and 7,9 N m (50 and 70 lbf in).

- Disconnect the nitrogen supply and remove the Turner Inflation Equipment T14218
and the Charging Adapter 460002502.

- Use the Turner Inflation Equipment T14218 and the Charging Adapter 460002502
to connect the nitrogen supply to the charging valve (13-60).

- Slowly increase the nitrogen pressure to between 6,90 and 8,27 bar (100 and
120 lbf/in2).

- Close the charging valve (13-60); use the Crowfoot Wrench T14500 to torque it to
between 5,7 and 7,9 N m (50 and 70 lbf in).

- Disconnect the nitrogen supply and remove the Turner Inflation Equipment T14218
and the Charging Adapter 460002502.

- Leakage Tests

- Make a record of the nitrogen pressure at the charging valve (13-60), (P1A) and
the charging valve (17-20), (P1B).

- Make a record of the ambient temperature, (T1).

- Keep the unit in this condition for a minimum of six hours.

- Measure the nitrogen pressure at the charging valve (13-60), (P2A) and the
charging valve (17-20), (P2B).

- Measure the ambient temperature, (T2).

- Compare the pressures P1A and P2A and compare the pressures P1B and P2B.
The pressures P1A and P2A must be the same and the pressures P1B and P2B
must be the same, unless:

- There is a difference between the temperatures T1 and T2

- There is an error because of the pressure gauge capacity.

- If there is a difference between the temperatures T1 and T2, calculate the correct
value for the nitrogen pressures (these will be P3A and P3B) and adjust the
pressures to the corrected values. Use the formula:

P3A = (P2A + Z) x (T1 + K) - Z

(T2 + K)
OR
(P2B + Z) x (T1 + K)
P3B =	- Z

(T2 + K)

Where K = 273 for temperatures in °C

(459 for temperatures in °F)

Z = 1 for pressures in bar

(15 for pressures in lbf/in2)

32-12-22

Page 104

Dec 6/2019

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

Main Landing Gear Leg (1-1)
Figure 101

32-12-22

Page 105

Dec 6/2019

sSAFRAN	SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

- If there is an error because of the gauge capacity:

- Close the charging valve (13-60).

- Release the pressure in the gauge.

3 Open the charging valve (13-60) and measure the nitrogen pressure, (P4A).

- Close the charging valve (13-60).

- Close the charging valve (17-20).

- Release the pressure in the gauge.

7 Open the charging valve (17-20) and measure the nitrogen pressure, (P4B).

- Calculate the correct values for the nitrogen pressures (these will be P5A and
P5B) and adjust the pressures to the corrected values. Use the formula:

P5A = P1A + (P2A - P4A)

OR

P5B = P1B + (P2B - P4B)

- Prepare for Transport and Storage

- Open the charging valve (13-60) and reduce the nitrogen pressure to between 3,45
and 4,82 bar (50 and 70 lbf/in2).

- Close the charging valve (13-60); use the Crowfoot Wrench T14500 to torque it to
between 5,7 and 7,9 N m (50 and 70 lbf in).

- Open the charging valve (17-20) and reduce the nitrogen pressure to between 3,45
and 4,82 bar (50 and 70 lbf/in2).

- Close the charging valve (17-20); use the Crowfoot Wrench T14500 to torque it to
between 5,7 and 7,9 N m (50 and 70 lbf in).

- Write this data on a label and attach it to the unit: THE GEAR MUST BE INFLATED
TO THE APPROPRIATE PRESSURES BEFORE BEING PLACED IN SERVICE.

- Complete the torque procedure for the retaining pins (13-10): refer to ASSEMBL .

- Proximity Switches (7-40 and 7-230) Adjustments and Tests

- Use the loading press: set the dimension between the pins (10-80 and 11-130) to
between 632,80 and 636,95 mm (24.9134 and 25.0767 in).

- Adjust the spacers (6-140, 7-50, 7-190 and 7-240) or laminated shims (6-140A, 7-50A,
7-90A and 7-240A): refer to ASSEMBLY.

NOTE: If the calculated gap is in the tolerance, the spacers (6-140, 7-50, 7-190
and 7-240) or laminated shims (6-140A, 7-50A, 7-90A and 7-240A) are not
necessary.

- Connect the 28 VDC power supply, the Lampbox 460005842 and the main landing gear
leg (1-1).

32-12-22

Page 106

Dec 6/2019

sSAFRAN	SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

- Use the loading press to fully extend the main landing gear leg (1-1).

- Use the loading press to slowly close the main landing gear leg (1-1):

- The proximity switch (7-230) must operate before the main landing gear leg (1-1)
has closed by 26,00 mm (1.0236 in).

- The proximity switch (7-40) must operate before the main landing gear leg (1-1)
has closed by 29,30 mm (1.1535 in).

- Do para (4) and (5) again.

- Disconnect the 28 VDC supply and the Lampbox 460005842.

- Remove the main landing gear leg (1-1) from the loading press.

- Electrical Bonding Resistance Tests (Refer to Figure 102)

CAUTION: DO NOT CAUSE DAMAGE TO THE PAINT FINISH.

NOTE: Make sure that the main landing gear leg (1-1) is electrically isolated from the
equipment that is used to hold it.

- Use the Milliohmmeter Megger, Type BT51, to measure the electrical bonding
resistance.

- Measure between the bearing (20-250) and the test points given in Table 101. The
electrical bonding resistance must not be more than the limit given in Table 101.

- Measure between the axle of the sliding tube subassembly (17-240) and the test
points given in Table 102. The electrical bonding resistance must not be more than
the limit given in Table 102.

32-12-22

Page 107

Dec 6/2019

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

BEARING (20-250)

3

8

AXLE OF SLIDING TUBE
SUBASSEMBLY (17-240)

A3219209-2

Electrical Bonding Resistance Test Points (Tables 101 and 102)
Figure 102

32-12-22

Page 108

Dec 6/2019

sSAFRAN	SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL

MAIN LANDING GEAR LEG

Electrical Bonding Resistance Tests

Table 101 (Refer to Figure 102)

| TEST POINT | IPL FIGURE AND ITEM No. | NAME | LIMIT VALUE MILLIOHMS |
| --- | --- | --- | --- |
| 1 | 2-80 | Bracket | 10 |
| 2 | 2-120 | Bracket | 10 |
| 3 | 4-100 | Proximity switch connector shell | 20 |
| 4 | 7-230 | Proximity switch connector shell | 20 |
| 5 | 7-100 | Harness support bracket | 20 |
| 6 | 10-160 | Upper pivot bracket | 20 |
| 7 | 7-40 | Proximity switch connector shell | 20 |
| 8 | 5-390 | Static discharge connector | 10 |

Electrical Bonding Resistance Tests
Table 102 (Refer to Figure 102)

| TEST POINT | IPL FIGURE AND ITEM No. | NAME | LIMIT VALUE MILLIOHMS |
| --- | --- | --- | --- |
| 9 | 11-140 | Harness support bracket | 20 |
| 10 | 8-170 | Bracket | 20 |

- Fault Isolation

A. To be given subsequently.

32-12-22

Page 109

Dec 6/2019

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

INTENTIONALLY BLANK

32-12-22

Page 110

sSAFRAN	SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG
DISASSEMBLY

- General

WARNING: DO NOT GET HYDRAULIC FLUID ON YOUR SKIN OR IN YOUR EYES. DO NOT
BREATHE THE FUMES. ONLY USE IN A LOCATION THAT HAS A CONTINUOUS
FLOW OF CLEAN AIR. HYDRAULIC FLUID IS POISONOUS AND DANGEROUS.

NOTE: Refer to TESTING AND FAULT ISOLATION to find the necessary level of
disassembly. This will give the condition of the component or the possible cause of
its malfunction.

- Make sure that the work area, the tools and the equipment are clean.

- Discard parts that you must not use again. These include:

| IPL Fig/Item No. | Part Name |
| --- | --- |
| 1-10 | Split pin |
| 1-41 | Heat shrink sleeve |
| 1-43 | Ferrule |
| 1-45 | Bowden cable |
| 1-80 | Tab washers |
| 2-70 | Tab washers |
| 2-150 | Tab washer |
| 2-180 | Tab washers |
| 2-200 | Tab washer |
| 2-230 | Split pins |
| 3-20 | Split pin |
| 3-180 | Split pin |
| 4-20 | Split pin |
| 4-110 | Split pins |
| 4-170 | Tab washers |
| 4-200 | Tab washers |
| 4-220 | Split pin |
| 4-270 | Split pins |
| 5-170 | Split pin |
| 5-320 | Split pins |
| 5-360 | Split pin |
| 6-60 | Split pin |
| 6-240 | Split pin |
| 7-60 | Split pins |
| 8-40 | Split pins |

32-12-22

Page 301

Dec 6/2019

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

| IPL Fig/Item No. | Part Name |
| --- | --- |
| 8-100 | Split pins |
| 9-10 | Split pin |
| 10-20 | Tab washers |
| 10-40 | Split pin |
| 10-100 | Tab washers |
| 10-150 | O-ring seals |
| 11-70 | Tab washers |
| 11-90 | Split pin |
| 12-10 | Split pins |
| 12-50 | Split pins |
| 12-100 | O-ring seal |
| 12-110 | Backing ring |
| 12-130 | O-ring seals |
| 12-140 | Backing rings |
| 12-150 | O-ring seals |
| 12-160 | Backing rings |
| 13-67 | O-ring seal |
| 13-120 | O-ring seal |
| 13-140 | Split pin |
| 14-20 | Split pins |
| 15-80 | Locking plate |
| 15-90 | Screws |
| 15-100 | Tab washers |
| 15-120 | Pin |
| 15-170 | Tab washers |
| 15-250 | O-ring seal |
| 15-260 | Backing rings |
| 15-280 | Seal |
| 15-290 | Seal |
| 15-310 | O-ring seal |
| 15-320 | Split pin |
| 16-20 | O-ring seal |
| 16-30 | Backing rings |
| 16-40 | O-ring seal |
| 16-50 | Backing rings |
| 16-60 | Seal |

32-12-22

Page 302

Mar 15/2024

sSAFRAN	SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

| IPL Fig/Item No. | Part Name |
| --- | --- |
| 16-70 | Seal |
| 16-80 | Joint seal |
| 16-90 | Sealing ring |
| 16-100 | Wiper ring |
| 16A-117 | Inner liner |
| 17-27 | O-ring seal |
| 17-60 | O-ring seal |
| 17-70 | Backing ring |
| 17-90 | Lock washer |
| 17-110 | Tab washers |
| 17-190 | Seal |
| 17-210 | O-ring seal |
| 17-220 | Backing ring |
| 19-10 | Split pin |

- Wire Thread Inserts

- If necessary, remove the wire thread inserts:

- Bend the outer coil of the wire thread insert to the centre of the hole.

- Remove the wire thread insert. Make sure that broken pieces do not stay in the
hole.

- Special Tools

- These special tools are necessary:

NOTE: Alternative equivalents are permitted.

| Part No. | Special Tool | Function |
| --- | --- | --- |
| MT1025 | Bench Clamp | Use with MT1026/63 and 460006406 |
| MT1026/63 | Holding Blocks | Hold the cylinder (17-230) |
| T14500 | Crowfoot Wrench | Remove the charging valves (13-60 and 17-20) |
| T14544 | Torque Adapter | Remove the nut (9-50) |
| 460001355 | Extractor | Remove the lubrication adapters (18-60), (20-130), (20-160), (20-190) and (20-220) |
| 460004331/1 | Drift | Use with 460006151/47 |
| 460004331/7 | Drift | Use with 460006151/24, 460006151/25, 460006151/26 and 460006151/51 |

32-12-22

Page 303

Mar 15/2024

SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

| Part No. | Special Tool | Function |
| --- | --- | --- |
| 460004331/8 | Drift | Use with 460006151/7, 460006151/20, 460006151/30 and 460006151/31 |
| 460004331/9 | Drift | Use with 460006151/47, 460006151/87 and 460006151/88 |
| 460004331/20 | Drift | Use with 460006151/21 and 460006151/22 |
| 460004331/21 | Drift | Use with 460006151/9, 460006151/10 and 460007259 |
| 460004680 | Extraction Tube | Remove the forward pintle bush (20-250A) |
| 460006151/7 | Extractor | Remove the bushes (18-30) |
| 460006151/9 | Extractor | Remove the bushes (20-340) and (20-350) |
| 460006151/10 | Extractor | Remove the bushes (10-250, 11-230, 18-40 and 20-330) |
| 460006151/20 | Extractor | Remove the bushes (18-20) |
| 460006151/21 | Extractor | Remove the bush (15-370) |
| 460006151/22 | Extractor | Remove the bush (15-380) |
| 460006151/24 | Extractor | Remove the bearings (5-280 and 5-290) and the bushes (20-380) |
| 460006151/25 | Extractor | Remove the bearing (4-340) and the bush (20-360) |
| 460006151/26 | Extractor | Remove the bearing (4-350) |
| 460006151/30 | Extractor | Remove the bush (6-210 and 8-160) |
| 460006151/31 | Extractor | Remove the bush (6-220 and 8-150) |
| 460006151/47 | Extractor | Remove the bearings (20-230, 20-240 and 20-290) |
| 460006151/48 | Extractor | Remove the bearing (20-290) |
| 460006151/51 | Extractor | Remove the bushes (20-390) |
| 460006151/86 | Extractor | Remove the bushes (20-320) |
| 460006151/87 | Extractor | Remove the bearing (20-270) |
| 460006151/88 | Extractor | Remove the bearing (20-260) |
| 460006208 | Lifting Bar Assembly | Hold the main landing gear leg (1-1), use with 460007281 and 460007282 |
| 460006211 | Lifting Tackle | Lift the sliding tube subassembly (17-240) and related parts |

32-12-22

Page 304

Mar 20/2020

sSAFRAN	SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

| Part No. | Special Tool | Function |
| --- | --- | --- |
| 460006213 | Transport and Build Trolley |  |
| 460006215 | Support Arms |  |
| 460006216 | Towing Frame | Hold the main landing gear leg (1-1) |
| 460006223 | Jacking Dome Adapter |  |
| 460006237 | Adapter |  |
| 460006232 | Extraction Pad | Remove the bearing (20-300) |
| 460006253 | Extractor | Remove the bearing (20-280) |
| 460006261 | Extraction Pad | Remove the bearing (20-310) |
| 460006262 | Extraction Bar | Use with 460006232, 460006263 and 460006261 |
| 460006263 | Extraction Pad | Remove the bearing (20-300) |
| 460006267 | Press Pad Assembly | Remove drag arm sleeve (20-370A) |
| 460006404 | Torque Adapter | Remove the jacking dome (17-80) |
| 460006406 | Holding Blocks | Hold the upper diaphragm tube (15-390) |
| 460006410 | Assembly/Extraction Tool | Remove the level tube (15-300) |
| 460006413 | Extractor | Remove the bushes (7-130) |
| 460006415 | Extractor Pad and Drawbolt | Remove the bush (2-310) |
| 460006416 | Extractor | Remove the bush (2-320) |
| 460006497 | Hydraulic-Pneumatic Pump Set | To remove the forward pintle bush |
| 460006498/7 | Bolt | (2-250A) |
| 460006499/25 | Press Pad |  |
| 460007230 | Torque Adapter | Remove the nuts (14-60) |
| 460007231 | Spacer | Use with 460007282 |
| 460007232 | Torque Adapter | Remove the locking nut (19-52) |
| 460007234 | Location Frame | Hold the main landing gear leg (1-1) (left configuration) |
| OR 460007235 | Location Frame | Hold the main landing gear leg (1-2) (right configuration) |
| 460007240 | Build Trolley | Hold the sliding tube subassembly (17-240) and related parts |

32-12-22

Page 305

Mar 20/2020

sSAFRAN	SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL

MAIN LANDING GEAR LEG

| Part No. | Special Tool | Function |
| --- | --- | --- |
| 460007242 | Torque Reaction Adapter | Hold the pin (9-70) |
| 460007254 | Extractor | Remove the bush (18-50) |
| 460007259 | Extractor Plate | Remove the bushes (20-340 and 20-350) |
| 460007278 | Torque Reactor | Use with 460006406 |
| 460007279 | Pin Spanner | Remove the upper bearing housing (15-40) |
| 460007281 | Pintle Location Assembly | Hold the main landing gear leg (1-1), |
| 460007282 | Spherical Bearing Locator | use with 460006208 |
| 460007283 | Torque Adapter | Remove the diaphragm subassembly (15-190) |
| 460007284 | Pin Spanner | Remove the nut subassembly (17-130) |

32-12-22

Page 306

Mar 20/2020

^SAFRAN	SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

- Procedure (Refer to IPL Figures 1 to 20)

- Initial Operations

- Use these special tools as necessary during the procedure to lift and to hold the unit:

- The Lifting Bar Assembly 460006208

- The Spherical Bearing Locator 460007282

- The Pintle Location Assembly 460007281

- The Transport and Build Trolley 460006213

- The Support Arms 460006215

- The Towing Frame 460006216

- The Jacking Dome Adapter 460006223

- The Adapter 460006237

- The Spacer 460007231

- The Location Frame 460007234 (for left configuration units)

- The Location Frame 460007235 (for right configuration units).

- Pintle Pin (1-60)

- Pre SB 201-32-22: remove the split pin (1-10), the nut (1-20), the washer (1-30), the bolt
(1-40) and the washer (1-50).

- Post SB 201-32-22: remove the split pins (1-10A), the nuts (1-20A), the spacers (1-25),
the washers (1-30A), the shims (1-35), the bolt subassembly (1-40A) and the washers
(1-50A).

- Post SB 201-32-22: cut the Bowden cable (1-45) and remove the cross bolts (1-47 and
1-49).

- Remove the pintle pin (1-60).

- Remove the bolts (1-70), tab washers (1-80), nuts (1-90) and retainers (1-100).

- Remove the wiring diagram plate (1-110).

- PRE SB 201-32-81: Transfer Block Subassembly (2-290 and 2-290A)

- Remove the bolt (2-10), the washer (2-20), the pin (2-30), the spacer (2-40) and the
threaded insert (2-50).

- Release the tab washers (2-70). Remove the bolts (2-60), the tab washers (2-70) and
the bracket (2-80).

- Remove the bolts (2-100), the washers (2-110) and the bracket (2-120).

- Release the tab washer (2-150). Remove the bolt (2-140), the tab washer (2-150) and
the bonding cable (2-160).

- Release the tab washers (2-180 and 2-200). Remove the bolts (2-170 and 2-190) and
the tab washers (2-180 and 2-200).

32-12-22

Page 307

Mar 19/2021

sSAFRAN	SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

- Remove the nuts (2-210) and retainers (2-220).

- Remove the split pins (2-230), the nuts (2-240), the washers (2-250), the bolts (2-260)
and the bracket (2-270).

- Remove the transfer block subassembly (2-290 and 2-290A).

- Use the Extractor Pad and Drawbolt 460006415 to remove the bush (2-310) and the
Extractor 460006416 to remove the bush (2-320) from the transfer block (2-340 and
2-340A).

- POST SB 201-32-81: Transfer Block Subassembly (2-290B)

- Remove the bolt (2-10), the washer (2-20), the pin (2-30), the spacer (2-40) and the
threaded insert (2-50).

- Release the tab washer (2-150). Remove the bolt (2-140), the tab washer (2-150) and
the bonding cable (2-160).

- Release the tab washers (2-180 and 2-200). Remove the bolts (2-170 and 2-190A) and
the tab washers (2-180 and 2-200).

- Remove the nuts (2-210) and retainers (2-220).

- Remove the transfer block subassembly (2-290B).

- Use the Extractor Pad and Drawbolt 460006415 to remove the bush (2-310) and the
Extractor 460006416 to remove the bush (2-320) from the transfer block (2-340B).

- Cardan Assembly (3-10)

- Remove the split pin (3-20), the nut (3-30), the washer (3-40), the pin (3-50), the spacers
(3-60 and 3-70) and the cardan assembly (3-10).

- Remove the lubrication fittings (3-80, 3-100 and 3-120) and the identification washers
(3-90, 3-110 and 3-130) from the lock stay cardan subassembly (3-140).

- Remove the bushes (3-150 and 3-160) from the lock stay cardan (3-170).

- Remove the split pin (3-180), the nut (3-190), the washer (3-200), the bolt (3-210) and
the bracket (3-220).

- Rod End Assembly (4-10), Proximity Switch (4-100) and Bracket Subassembly (4-330)

- Remove the split pin (4-20), the nut (4-30), the washer (4-40) and the rod end assembly
(4-10).

- Remove the spherical bearing (4-50) from the rod end (4-60).

- Remove the nuts (4-70), the washers (4-80), the cap screws (4-90) and the proximity
switch (4-100).

- Remove the split pins (4-110), the nuts (4-120), the washers (4-130) and the bracket
(4-140).

- Release the tab washers (4-170 and 4-200). Remove the nuts (4-160), the tab washers
(4-170), the spacer (4-180), the bolts (4-190), the tab washers (4-200) and the bracket
(4-210).

- Remove the split pin (4-220), the nut (4-230), the washer (4-240), the bolt (4-250) and
the sleeve (4-260).

32-12-22

Page 308

Mar 19/2021

sSAFRAN	SAFRAN LANDING SYSTEMS UK Ltd

CAGE: K0654

PART No. 201587001 AND 201587002 COMPONENT MAINTENANCE MANUAL
MAIN LANDING GEAR LEG

- Remove the split pins (4-270), the nuts (4-280), the washers (4-290) and the bolts
(4-300).

- Remove the bonding cable (4-310) and the washer (4-320).

- Remove the bracket subassembly (4-330).

- Use the Drift 460004331/7 and the Extractor 460006151/25 to remove the bearing
(4-340) from the bracket (4-360).

- Use the Drift 460004331/7 and the Extractor 460006151/26 to remove the bearing
(4-350) from the bracket (4-360).

- Bracket Assembly (5-10), Bracket Subassemblies (5-90 and 5-270) and Uplock Pin (5-400)

- Remove the bolts (5-20 and 5-40), the washers (5-30 and 5-50), the nut (5-60) and the
bracket assembly (5-10).

- Remove the spherical bearing (5-70) from the bracket (5-80): refer to M-DLPS1014-2.

- Remove the bolts (5-100 and 5-120), the washers (5-110 and 5-130), the nut (5-140)
and the bracket subassembly (5-90).

- Remove the spherical bearing (5-150) from the bracket (5-160): refer to M-DLPS1014-2.

- Remove the split pin (5-170), the nut (5-180), the washer (5-190), the bolt (5-200) and
the sleeve (5-210).

- Remove the bolt (5-220), the washer (5-230) and the bonding cable (5-240).

- Remove the bolt (5-250), the washer (5-260), the bracket subassembly (5-270) and the
nut (5-310).

- Use the Drift 460004331/7 and the Extractor 460006151/24 to remove the bearings
(5-280 and 5-290) from the bracket (5-300).

- PRE SB 201-32-72:

- Remove the split pins (5-320), the nuts (5-330), the washers (5-340) and the bolts
(5-350).

- Remove the split pin (5-360), the nut (5-370), the washer (5-380), the static
discharge connector (5-390) and the uplock pin (5-400).

- POST SB 201-32-72:

- Remove the split pins (5-320A), the nuts (5-330A), the washers (5-340B) and the
bolts (5-350A).

- Remove the ground stud subassembly (5-390A), the washer (5-380B), the
bolt (5-395) and the uplock pin (5-400A).

- Slave Link Subassembly (6-190) and Lower Slave Link Subassembly (6-290)

- Remove the split pin (6-10), the nut (6-20), the washer (6-30), the bolt (6-40) and the
spacer (6-50).

- Remove the split pin (6-60), the slotted nut (6-70), the washer (6-80) and the pivot pin
(6-90).

- Remove the slave link subassembly (6-190) and its attached parts.

32-12-22

Page 309

Mar 20/2020